| EXAMPLE |  |  | SOLDIER PILE BOX |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Soil | active soil | trench | equipment | top strut | bot strut | pile | saftey | Pile | Wale |
| coefficient | loading | depth | surcharge | from top | from bot | allow | factor | spacing | spacing |
| Ka | Ga | H | Ge | St | Sb | Fy | Sf | B | Sc |
| factor | lb/cft | ft | lb/sf | ft | ft | psi | ratio | ft | ft |
| 0.80 | 33.3 | 40.00 | 72 | 11.50 | 11.50 | 36,000 | 1.5 | 7.5 | 17.00 |
| 0.2H | St | Ws = KaGaH |  |  |  |  |  |  |  |
|  | Qt | Wt = [0.8Ws+Ge]H |  |  | Fa | Ws | Wt | Qt | Qb |
| H |  | Qt = Wt(H/2-Sb)/Sc |  |  | psi | lb | lb | lb/lf | lb/lf |
| Wt | Sc | Qb = Wt-Qt |  |  | 24,000 | 1,066 | 36,979 | 18,490 | 18,490 |
|  | Qb |  |  |  |  |  |  |  |  |
| 0.2H | Sb | X = [Qt -WsH/10-GeH/5]/[Ws+Ge], Shear = Zero, Moment maximum |  |  |  |  |  |  |  |
|  |  | Mt = B[WsH(St-H/7.5)/10+Ws(St-0.2H)^2/2+GeSt^2/2} |  |  |  |  |  |  |  |
|  |  | Mc = B[Qt(X+0.2H-St)-WsH(X+H/15)/10-WsX^2/2-Ge(X+0.2H)^2/2] |  |  |  |  |  |  |  |
|  |  | Mb = B[WsH(Sb-H/7.5)/10+Ws(Sb-0.2H)^2/2]+GeSb^2/2 |  |  |  |  |  |  |  |
|  |  | Zx, min = 12Mmax/Fa |  |  |  |  |  |  |  |
| X | Mt | Mc | Mb | Mmax | Zx, min | Select | Pile |  | Pile width |
| ft | ft-lb | ft-lb | ft-lb | ft-lb | cin | Pile | Zx, cin |  | Bf, inch |
| 12.00 | 281,795 | 26,424 | 281,795 | 281,795 | 140.90 | HP14x89 | 146 | OK | 14.695 |
| SHEETING |  |  |  |  |  |  |  |  |  |
|  | STEEL |  | OK |  |  |  |  |  |  |
| Use: | t = plate thickness = |  | 1 | in |  | WOOD: |  |  |  |
| Min: | t = (Sx/2)^0.5 = |  | 0.93 | in |  | Use: |  | 4 | ? x 12 in |
|  | Fp=Plate steel Grade |  | 36,000 | psi | Min: | d=(6Sx/b)^0.5,b=12" |  | 4.0 | inch |
|  | Sp = Safety factor |  | 1.33 |  |  | fa = wood bending= |  | 1,500 | psi |
|  | Wp = 0.6GaH+Ge = |  | 799 | lb/sf |  | Sx,wood=12*m/fa = |  | 31.47 | cin |
|  | Mp = W[B-Bf/12]^2/8 = |  | 3,934 | ft-lb |  |  |  |  |  |
|  | Fa = Fy/Sp = |  | 27,068 | psi |  | Use | Plate | or Wood |  |
|  | Sp, steel = 12Mp/Fa = |  | 1.74 | cin |  |  | Thickness | Thickness |  |
|  | Ix = t^4 = |  | 1.000 | in^4 |  |  | inch | inch |  |
| deflection | d = 5Wp(B-Bf)^4/384EI |  | 0.93 | in |  |  | 1 | 4 |  |
| DESIGN COFFERDAM WALES |  |  |  | Length, Ls = | 31.00 | ft |  | Length |  |
|  |  |  |  | Width, Lj = | 21.00 | ft |  |  |  |
| UPPER | shield | Beam | M, ft-lb |  |  |  |  |  |  |
| WALE | length | Span | QtLb^2/8 | P |  |  |  |  |  |
| LENGTH | Ls | Lb=Ls-Dj | -Vdi/24 | QtLj/2 | Es | Width |  |  |  |
|  | ft | ft | ft-lb | lb | psi |  |  |  |  |
|  | 31.00 | 29.64 | 1,914,497 | 194,141 | 2.90E+07 |  |  |  |  |
| Beam size | Zx, cin | A, sqin | Depth, Di in | r(x), in | Web Thick | Stl Grade | Sf: |  |  |
| W14x257 | 487.0 | 74.60 | 16.38 | 6.71 | 1.175 | 36,000 | 1.5 |  |  |
| Cc =[2Pi^2E/Fy]^0.5 |  | KL/r | Ca | C | Ba | B |  |  |  |
| 126.10 |  | 55.44 | 17,860 | 2,602 | 24,000 | 47,174 |  |  |  |
| OK |  |  | 1>C/Ca+B/Ba |  | 2.11 | NO |  |  |  |
|  |  | Beam |  | Mmax |  |  |  |  |  |
| UPPER | Span | Span | Qt | QtLb^2/8 | P | V |  |  |  |
| WALE | Width, Lj | Lb=Lj-2Di |  | -PDj/36 | QtLs/2 | QtLb/2 | Es |  |  |
| WIDTH | ft | ft | lb/lf | ft-lb | lb | lb | psi |  |  |
|  | 21.00 | 18.27 | 18,490 | 641,064 | 286,589 | 168,902 | 2.90E+07 |  |  |
| Beam size | Zx, cin | A, sqin | Depth, Dj in | r(x), in | Web Thick | Stl Grade | Sf: |  |  |
| W14x257 | 487.0 | 74.60 | 16.38 | 6.71 | 1.175 | 36,000 | 1.5 |  |  |
| Cc =[2Pi^2E/Fy]^0.5 |  | KLw/r | Ca | C | Ba | B |  |  |  |
| 126.10 |  | 32.67 | 19,749 | 3,842 | 24,000 | 15,796 |  |  |  |
| OK |  |  | 1>C/Ca+B/Ba |  | 0.85 | OK |  |  |  |
| WALE SPLICE |  | V = W[Lj-Di/6\/2 = |  | 168,902 | lbs |  |  |  |  |
| A325 bolts, single shear, "N" STD (threads in shear plane) |  |  |  |  |  |  |  |  |  |
|  | Bolt Size | Bolt | Fv = Bolt | Pb = Bolt | Min No. | Use No. |  |  |  |
|  | Diameter | Area | Allowable | Allowable | of Bolts | of bolts |  |  |  |
|  | inch | sqin | Stress, psi | Load, lbs | each | each |  |  |  |
|  | 1 3/8 | 1.485 | 21,000 | 31,183 | 5.42 | 6 |  |  |  |
| Shear plate |  |  |  |  |  |  |  |  |  |
|  | Weld plate to beam w/ fillet all around |  |  |  |  |  |  |  |  |
| Steel | Safety | Size of | No. of | Weld | Lw=Weld | Weld | Plate min |  |  |
| Grade | factor | butt | Welds | Stress | length, Lw | Load | Thickness |  |  |
| Fy, psi | Sf | Weld, in | each | 0.6Fy, psi | Dj-2, inch | lbs | inch |  |  |
| 36,000 | 1.65 | 5/8 | 1 | 21,600 | 14.38 | 194,130 | 0.76346683 |  |  |
|  |  |  |  |  |  | OK | 0.6875 |  |  |
| WEB BEARING |  |  |  |  |  |  |  |  |  |
|  | Bearing = largest Q = |  |  | 286,589 | lbs |  |  |  |  |
|  | Bearing stress = Q/(web length x web thickness); 0.75Fy psi maximum |  |  |  |  |  |  |  |  |
|  | W14x257 | Dj, inch | tw, inch | Stress |  |  |  |  |  |
|  |  | 16.38 | 1.175 | 14,890 | psi | OK |  |  |  |
| LOWER | shield | Beam |  | M, ft-lb |  |  |  |  |  |
| WALE | length | Span |  | QbLi^2/8 | P |  |  |  |  |
| LENGTH | Ls | Li=Ls-Dj | Qb | -VDj/24 | QtLj/2 | Es |  |  |  |
|  | ft | ft | lb/ft | ft-lb | lb | psi |  |  |  |
|  | 31.00 | 27.94 | 18,490 | 1,593,470 | 194,141 | 2.90E+07 |  |  |  |
| Beam size | Zx, cin | A, sqin | Depth, Di in | r(x), in | Web Thick | Stl Grade | Sf: |  |  |
| W36x300 | 1260 | 88.3 | 36.74 | 15.2 | 0.945 | 36,000 | 1.5 |  |  |
| Cc =[2Pi^2E/Fy]^0.5 |  | KL/r | Ca | C | Ba | B |  |  |  |
| 126.10 |  | 24.47 | 20,317 | 2,199 | 24,000 | 15,176 |  |  |  |
| OK |  |  | 1>C/Ca+B/Ba |  | 0.74 | OK |  |  |  |
|  |  |  |  | Mmax |  |  |  |  |  |
| LOWER | Span | Wale Length | Qb | QbLw^2/8 | V | P | Es |  |  |
| WALE | Width, Lj | Lw=Lj-Di/6 |  | -PDj/36 | QbLw/2 | QbLs/2 | Modulus |  |  |
| WIDTH | ft | ft | lb | ft-lb | lb | lb | psi |  |  |
|  | 21.00 | 14.88 | 18,490 | 219,024 | 137,532 | 286,589 | 2.90E+07 |  |  |
| Beam size | Zx, cu in | A, sqin | Depth, Dj in | r(x), in | Web Thick | Stl Grade | Sf: |  |  |
| W36x300 | 1260 | 88.3 | 36.74 | 15.2 | 0.945 | 36,000 | 1.5 |  |  |
| Cc =[2Pi^2E/Fy]^0.5 |  | KLw/r | Ca | C | Ba | B |  |  |  |
| 126.10 |  | 11.74 | 21,066 | 3,246 | 24,000 | 2,086 |  |  |  |
| OK |  |  | 1>C/Ca+B/Ba |  | 0.24 | OK |  |  |  |
| WALE SPLICE |  | V = |  | 137,532 | lbs |  |  |  |  |
| A325 bolts, single shear, "N" STD (threads in shear plane) |  |  |  |  |  |  |  |  |  |
|  | Bolt Size | Bolt | Fv = Bolt | Pb = Bolt | Minimum No. |  |  |  |  |
|  | Diameter | Area | Allowable | Allowable | of Bolts | Use |  |  |  |
|  | inch | sqin | Stress, psi | Load, lbs | each | each |  |  |  |
|  | 1 1/4 | 1.227 | 21,000 | 25,771 | 5.34 | 8 |  |  |  |
| Shear plate |  |  |  |  |  |  |  |  |  |
|  | Weld plate to beam w/ fillet all around |  |  |  |  |  |  |  |  |
| Steel | Safety | Size of | No. of | Weld | Lw=Weld | Weld | Plate min |  |  |
| Grade | factor | Butt | Welds | Stress | length, Lw | Load | Thickness |  |  |
| Fy, psi | Sf | Weld, in | each | Fy, psi | Dj-2, inch | lbs | inch |  |  |
| 36,000 | 1.65 | 3/8 | 1 | 18,000 | 34.74 | 234,495 | 0.574 |  |  |
|  |  |  |  |  |  | OK | 5/8 |  |  |
| WEB BEARING |  |  |  |  |  |  |  |  |  |
|  | Bearing = largest Q = |  |  | 286,589 | lbs |  |  |  |  |
|  | Bearing stress = Q/(web length x web thickness); 0.75Fy psi maximum |  |  |  |  |  |  |  |  |
|  | W36x300 | Dj, inch | tw, inch | Stress |  |  |  |  |  |
|  |  | 36.74 | 0.945 | 8,254 | psi | OK |  |  |  |