| EXAMPLE |  |  |  | SOLDIER PILE BRACED |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Acive | Active | Passive | Passive |  |  |  |  |  |
| Soil | Friction | Soil | Friction | Active soil | Trench | Wale Set | Pile | Surcharge |
| Density | Angle | Density | Angle | Constant | Depth | from top | Spacing | Pressure |
| Gs | Fs | Gd | Fp | K | H | St | B | Ge |
| lb/cf | degrees | lb/cf | degrees | ? : 1 | ft | ft | ft | psf |
| 100 | 30 | 100 | 30 | 0.8 | 29.25 | 11.50 | 8.25 | 72 |
| Active | Passive | Active soil | Passive soil | Ca = [1-SinFs]/[1+SinFs] |  | Steel | Safety | Allowable |
| Factor | Soil Factor | pressure | Resistance | Cp = [1+SinFp]/[1-SinFp] |  | Yield | factor | Bending |
| Ca | Cp | Ga | Gp | Ga = CaGs |  | Fy | Sf | Fa |
| ? : 1 | ? : 1 | lb/cft | lb/cft | Gp = CpGd |  | psi | ? : 1 | psi |
| 0.33 | 3.00 | 33.33 | 300 | Fs, radians = | 0.5236 | 50,000 | 1.5 | 33,333 |
|  |  |  |  | Fp, radians = | 0.5236 |  |  |  |
| Note: Goal Seek "Y" until Msum = 0.00; (conservation of moments) |  |  |  |  |  |  |  |  |
| Msum = 0 | Y | Mcant | Mx | Zxmin = | Pile | Pile | Pile | flange |
| ft-lb | ft | ft-lb | ft-lb | cin | Use Size | Zx, cin | Ix, in^4 = | Bf, inch |
| 0.00 | 10.46 | 285,039 | 27,131 | 102.61 | W14x90 | 157 | 999 | 14.52 |
|  |  |  |  |  |  |  | OK |  |
| Dp | Ws | W | Qt | R | X, ft | Lp | Dx |  |
| ft | lb/ft | lb | lb | lb | ft | ft | inch |  |
| 2.42 | 6,435 | 167,954 | 128,257 | 39,697 | 15.07 | 39.71 | 1.24 |  |
|  |  |  |  | Dp = Bf/6 |  |  |  |  |
|  |  |  |  | Ws = KGaBH |  |  |  |  |
|  | 0.2H | St |  | W = [0.8KGaH +Ge]HB |  |  |  |  |
|  |  | Qt |  | R = GpDpY^2/2 |  |  |  |  |
| H | H |  |  | Qt =W - R |  |  |  |  |
|  | W | X |  | Msum = 0 = W(H/2-St)-R(H+2Y/3-St) |  |  |  |  |
|  |  |  |  | X = [Qt- WsH/10 -0.2GeBH]/ [BGe+Ws)] |  |  |  |  |
|  | 0.2H |  |  | Mx = Qt(X+0.2H-St)-BGe(X+0.2H)^2/2-WsX^2/2-WsH(X+H/15)/10 |  |  |  |  |
|  |  |  |  | Mcant = WsH(St-H/7.5)/10+Ws(St-0.2H)^2/2+GeBSt^2/2] |  |  |  |  |
| Y |  | R |  | Sx = 12Mmax/Fa |  |  |  |  |
|  |  |  |  | Lp = H+Y |  |  |  |  |
|  |  |  | Pile Deflection: | Dx =5(W/H/12)(12(H+Y/2-St))^4/(384EI) |  |  |  |  |
| Fa = Allowable bending stress, Fy/Sf |  |  |  |  | Lp = Minimum sheet pile length |  |  |  |
| B = soldier pile spacing |  |  |  |  | Mmax = Maximum bending moment |  |  |  |
| Dp = passive pile influence width |  |  |  |  | Msum = 0 = Conservation of moments |  |  |  |
| Esteel = Modulus of elasticity = 29,000,000 psi |  |  |  |  | Mx = Bending moment at X, shear = 0 |  |  |  |
| Ewood = Modulus of elasticity = 1,760,000 psi |  |  |  |  | R = Total soil passive pressure |  |  |  |
| Fy = steel yield stress |  |  |  |  | St = Ground to wale heigth |  |  |  |
| Ga = Active soil pressure |  |  |  |  | Sx = Section modulus |  |  |  |
| Gp = Passive soil pressure |  |  |  |  | W = Total soil pressure per pile |  |  |  |
| H = Excavation depth |  |  |  |  | X = Heigth to zero shear below triangle block |  |  |  |
| Ix =Moment of Inertia, in^4 |  |  |  |  | Y = Sheet pile toe penetration |  |  |  |
| Lagging | STEEL: |  |  |  | WOOD: |  |  |  |
|  | Fy = |  | 36,000 | psi | fa = |  | 1,500 | psi |
|  | B = |  | 8.25 |  | Sx, WOOD = 12*m/ba = |  | 32.56 | cin |
|  | W =0.75KGaH+Ge = |  | 657 | lb/sf | t = (6Sx/b)^0.5, b =12" |  | 4.0 | in |
|  | M = W(B-Bf/12)^2/8 = |  | 4,070 | ft-lb |  |  |  |  |
|  | Fa = 0.75*Fy = |  | 27,000 | psi |  | USE: |  |  |
|  | Sx, steel = 12M/Fa = |  | 1.81 | cin |  | Plates or | ?x12 Timber |  |
|  | t = (Sx/2)^0.5 |  | 0.951 | in |  | inch | inch |  |
| deflection | d = 5W(B-1)^4/384EI |  | 0.70 | in |  | 1 | 4 |  |
| Wale design: Use "H" shape -strut |  |  |  |  |  |  |  |  |
|  | I B I | B I | B I | B I | B I |  |  |  |
|  | ut |  |  |  | ut |  |  |  |
|  | str | N = | 3 | ea | str |  |  |  |
|  |  | L =NB | 24.75 | ft |  |  |  |  |
|  | I I | I | I | I | I |  |  |  |
| WALE | Zxmin = 12M/Fb = |  | 380.92 | cin | Qt = |  | 128,257 | lbs |
|  | Try: | W14x211 |  |  | Vwale = |  | 128,257 | lbs |
|  | Zx: | 390 | cin | OK | Aweb = td |  | 15.41 | sqin |
|  | Ix: | 2,660 | in^4 | OK | Vweb = Vwale/Aweb |  | 8,325 | psi |
|  | d, depth: | 15.72 | in |  | Mc = BP = |  | 1,058,118 | ft-lb |
|  | t, web: | 0.98 | in |  | Fb=Fy/Sf: |  | 33,333 | psi |
|  | k, flange | 2.25 | in |  | Mmax = |  | 1,058,118 | ft-lb |
|  | Stl Grade | 50,000 | psi |  | Es = |  | 29,000,000 | psi |
|  | Sf: | 1.5 |  |  | Dx = PB^3/[3EI] = |  | 0.54 | in |
| STRUT |  | Ps = NQt = | 384,770 | lb | Cc =[2Pi^2Es/Fy]^0.5 = |  | 107.00 |  |
|  | Try: | W14x145 |  |  | KL/r = |  | 36.18 | OK |
|  | Span = | 12 | ft |  | Fa = (compression) = |  | 26,356 | psi |
|  | ry = | 3.98 | in |  | Pstrut/ Area = |  | 9,011 | psi |
|  | Zx = | 260 | in |  | Mb = PsDb/2 = |  | 2,843,452 | in-lb |
|  | Db = | 14.78 | in |  | Fbh = Mb/Zx = |  | 10,936 | psi |
|  | Zy = | 133 | cin |  | Mv = 12wL^2/8 = |  | 64,800 | ft-lb |
|  | Area = | 42.7 | sqin |  | Fbv (bending) = M/Sy |  | 487 | psi |
|  | live load w = | 300 | lb/lft |  | Fba = |  | 33,333 | psi |
|  |  | Check: | Ps/Fca+Fbv/Fba +Fbh/Fba< 1 |  |  | 0.68 | OK |  |
| STRUT BEARING |  | W14x145 |  | Minimum Weld Load Qw = Ps(Bf-2k)/Bf = |  |  | 297,887 | lbs |
| Q = | Ca = | Flange, Tf | Flange, Bf | Beam k | Flange Area |  | Weld Size |  |
| 384,770 | 33,333 | 1.09 | 15.5 | 1.75 | 16.90 | OK | 5/8 |  |
| stiffners | Stiff D | Stiff T | Stiff Area | Area min | Allowable Weld Load Qa = 10800Wz2Ns(Dw-2K) |  |  |  |
| 2 | 7.75 | 1.00 | 15.50 | 11.54 | OK | 302,940 | OK |  |