| EXAMPLE |  |  | CANTILEVER SOLDIER PILES |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Gs, Soil | Af, soil | H, Bank | Ge, Live | B, Pile | S, bank | Fy, Steel | Sf, Safety | Es, steel |
|  | density | Friction | height | loading | spacing | slope | yeild | factor | modulus |
|  | lb/cft | angle, deg | ft | lb/sf | ft | ?H : 1V | psi | ? : 1 | psi |
| active = | 100 | 30 | 12.50 | 72 | 8.25 | 100.00 | 36,000 | 1.50 | 2.90E+07 |
| Passive = | 100 | 30 |  |  |  |  |  |  |  |
|  |  | Af, soil | Ca, Active | Ap, soil | Cp,passive |  |  | Fa, Steel | Mmax |
|  |  | Active | coefficient | passive | coefficient | Ga | Gp | allowable | moment |
|  |  | angle, rad | ? : 1 | angle, rad | ? : 1 | lb/cft | lb/cft | psi | ft-lb |
|  |  | 0.52 | 0.33 | 0.524 | 3.00 | 33.33 | 300 | 24,000 | 307,919 |
| Ca = [1-sinAf]/[1+sinAf] |  |  |  |  |  |  |  |  |  |
| Cp = [1+sinAf]/[1-sinAf] |  | Dp | N | P | R | Q | Check | X | Z |
| Ga = CaGs |  | ft | lb | lb | lb | lb | V = 0 | ft | ft |
| Gp = CpGs |  | 2.42 | 7,425 | 21,484 | 141,360 | 112,451 | 0.000 | 8.92 | 5.96 |
| Soldier | PILE: | Bf, flange | Ix, pile | Lp, pile | Dx, pile |  |  |  |  |
| Pile | W14x90 | width | inertia | min length | deflection |  |  |  |  |
| Zxmin, cin | Zx, cin | inch | in^4 | ft | inch | Goalseek, Msum=0, by changing Y |  |  | Y, ft |
| 153.96 | 157 | 14.520 | 999 | 32.23 | 0.63 |  | Msum = 0 | 0 | 19.7 |
|  | OK |  |  |  |  |  |  |  |  |
| Mmax @ v=0 = N(H/2+X) + P(H/3+X) - RX/3 |  |  |  |  |  |  |  |  |  |
| Msum = 0 =N[H/2+D]+P[H/3+D]-RD/3+QY/3 |  |  |  |  |  | B = Soldier pile spacing c-c, ft. |  |  |  |
|  |  |  |  |  |  | Bf = Pile flange width, inches |  |  |  |
|  |  |  | Dp = Bf/6 |  |  | Dp = Pile passive influence width, ft |  |  |  |
|  |  |  | N = GeHB |  |  | Fa = Allowable steel bending stress, psi |  |  |  |
|  | N | H | P = GaBH^2/2 |  |  | Fy = Allowable bending stress, Gr50/1.4 S.F |  |  |  |
|  |  |  | R = DpGpD^2/2 |  |  | Ga = Active soil pressure, equivalent fluid |  |  |  |
|  | P |  | Q = GpDp(H+2D)Z/2 |  |  | Gp = Passive soil pressure |  |  |  |
|  |  |  | Z = 2[R-N-P]/[GpDp(H+2D)] |  |  | Gs = Soil Unit Weight |  |  |  |
|  |  |  | X = [2(N+P)/(GpDp)]^0.5 |  |  | H = Cantilever height |  |  |  |
|  | X |  | Lp = H+D |  |  | Ix = Moment of inertia |  |  |  |
| Y |  |  |  |  |  | Lp= Minimum pile length |  |  |  |
|  |  | R |  |  |  | Mmax = Maximum bending moment |  |  |  |
| Q |  | Z | Zxmin = Mmax*12/A |  |  | Msum = 0 = Conservation of moments |  |  |  |
|  |  |  |  |  |  | N = Surecharge live loading |  |  |  |
| Deflection, Dx = PaLd^3/(15EI)+(Ge/12)Ld^4/(8EI) |  |  |  |  |  | P = Total active soil force above invert |  |  |  |
| Ld= (H+0.7X)12 = |  | 225 | inches |  |  | Q = Tip passive reaction force |  |  |  |
|  |  |  |  |  |  | R = Soil passive pressure reaction force |  |  |  |
|  |  |  |  |  |  | X = heigth from O.G. @ Mmax, ft |  |  |  |
|  |  |  |  |  |  | Y = Sheet pile toe embedment, ft |  |  |  |
|  |  |  |  |  |  | Zx = Pile plasticn modulus |  |  |  |
| Lagging | STEEL: |  |  |  |  |  |  |  |  |
|  | W = 0.75*Ga*H+Ws = |  | 385 | lb/sf |  |  |  |  |  |
|  | M = W[B-1]^2/8 = |  | 2,526 | ft-lb |  |  |  |  |  |
|  | Fy = |  | 36,000 | psi |  |  |  |  |  |
|  | Fa = 0.75Fy = |  | 27,000 | cin | Lagging | WOOD: | Lw = | 7.25 | ft |
|  | Sx, steel = 12M/Fa = |  | 1.12 | in |  | Fw = |  | 1,500 | psi |
|  | t = [Sx/2]^1/2 |  | 0.749 | in |  | Sx, Wood = 12*M/Fw = |  | 20.21 | cin |
|  | Ix = t^3 = |  | 1 | in |  | t = [Sx/2]^1/2 |  | 3.2 | in |
|  | d = 5W(B-1)^4/384EI |  | 0.82 | in | Use: | 1 | "plate or | 4 | x12 wood |