| EXAMPLE |  |  | SHEET PILE BOX |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| trench | top strut | bot strut | Soil | Friction | pile | equipment | Soil | saftey | active soil |  |
| depth | from top | from bot | Density | Angle | allow | surcharge | coefficient | factor | loading |  |
| H | St | Sb | Gs | F | Fy | Ge | Ka | Sf | Ga |  |
| ft | ft | ft | pcf | degrees | psi | lb/sf | factor | ratio | lb/cft |  |
| 40.00 | 10.00 | 10.00 | 100 | 30 | 50,000 | 72 | 0.80 | 1.5 | 33.3 |  |
| 0.2H | St |  | Ga = Gs(1-SinF)/(1+SinF) |  |  |  | Zx, cin | Sheet Pile | Pile Sx |  |
|  | Qt |  | Ws = KaGaH |  |  |  | 7.25 | PZ22 | 18.1 |  |
| H |  |  | Wt = [0.8Ws+Ge]H |  |  |  |  |  | OK |  |
| Wt | Sc |  | Qt = Wt(H/2-Sb)/(H-Sb-St) |  |  |  |  |  |  |  |
|  | Qb |  | Qb = Wt-Qt |  |  |  |  |  |  |  |
| 0.2H | Sb |  | X = [Qt-WsH/10-GeH/5]/[Ws+Ge] |  |  |  |  |  |  |  |
|  |  |  | Mt = 0.75[WsH(St-H/7.5)/10+Ws(St-0.2H)^2/2]+GeSt^2/2 |  |  |  |  |  |  |  |
|  |  |  | Mc = 0.75[Qt(X+0.2H-St)-WsH(X+H/15)/10-WsX^2/2]-Ge(X+0.2H)^2/2 |  |  |  |  |  |  |  |
|  |  |  | Mb = 0.75[WsH(Sb-H/7.5)/10+Ws(Sb-0.2H)^2/2]+GeSb^2/2 |  |  |  |  |  |  |  |
| Ws | Wt | Qt | Qb | Fa | X | Mt | Mc | Mb | Mmax |  |
| psf | lb/lf | lb/lf | lb/lf | psi | ft | ft-lb | ft-lb | ft-lb | ft-lb |  |
| 1,067 | 37,013 | 18,507 | 18,507 | 33,333 | 12.00 | 20,133 | 19,867 | 20,133 | 20,133 |  |
| DESIGN COFFERDAM WALES |  |  |  | Nominal Length, Ls = | 30.00 | ft | Len. | gth. |  |  |
|  |  |  |  | Nominal Width, Lj = | 18.00 | ft |  |  |  |  |
|  | Sheet Pile: |  |  | PZ22 |  |  | 30.58 | ft |  |  |
|  | C = Corner Length = |  |  | 7.5 | in |  |  |  |  |  |
|  | Lp = Pile Length = |  |  | 22 | in |  |  |  |  |  |
|  | Ne = No. of Piles length = |  |  | 16 | each | Width |  |  | 19.58 | ft |
|  | Ls = Length = [2C+NeLp]/12 = |  |  | 30.58 | ft |  |  |  |  |  |
|  | Nw = No. of Piles Width = |  |  | 10 | each |  |  |  |  |  |
|  | Lj = Width = [2C+NwLp]/12 = |  |  | 19.58 | ft |  |  |  |  |  |
| UPPER | shield | Beam | M = RLb/2-QtLs^2/4-Vdi/24 |  |  |  |  |  |  |  |
| WALE | length | Span |  |  | R | V | P |  |  |  |
| LENGTH | Ls | Lb=Ls-Dj/12 | M, ft-lb | Qt | QtLs/2 | QtLm/2 | QtLj/2 |  |  |  |
|  | ft | ft | ft-lb | lb/ft | lb | lb | lb |  |  |  |
|  | 30.58 | 28.08 | 1,621,645 | 18,507 | 282,998 | 125,845 | 181,211 |  |  |  |
| Beam size | Zx, cin | A, sqin | Depth, Di in | r(x), in | Web Thick | Fy, psi | Sf: |  |  |  |
| W36x230 | 943 | 67.6 | 35.9 | 14.9 | 0.76 | 36,000 | 1.5 |  |  |  |
|  |  |  | Ca =Fy[1-K^2/2Cc^2]/[5/3+3K/8Cc-K^3/8Cc^3], Cc >K |  |  |  |  |  |  |  |
| Cc =[2Pi^2Es/Fy]^0.5 |  | K =12Lw/rx | Ca | C = P/A | Ba=Fy/Sf | B=12M/Zx | Es |  |  |  |
| 126.10 |  | 24.63 | 20,307 | 2,681 | 24,000 | 20,636 | psi |  |  |  |
| OK |  |  |  | 1>C/Ca+B/Ba = | 0.99 | OK | 2.90E+07 |  |  |  |
|  |  | Beam |  |  |  |  |  |  |  |  |
| UPPER | Span | Span | Qt | Mmax | P | V |  |  |  |  |
| WALE | Width, Lj | Lm=Lj-Di/6 |  | QtLm^2/8 | QtLs/2 | QtLm/2 | Es |  |  |  |
| WIDTH | ft | ft | lb/lf | ft-lb | lb | lb | psi |  |  |  |
|  | 19.58 | 13.60 | 18,507 | 427,874 | 282,998 | 125,845 | 2.90E+07 |  |  |  |
| Beam size | Zx, cin | A, sqin | Depth, Dj in | r(x), in | Web Thick | Stl Grade | Sf: |  |  |  |
| W30x116 | 378 | 34.2 | 30.01 | 12 | 0.565 | 36,000 | 1.5 |  |  |  |
|  |  |  | Ca =Fy[1-K^2/2Cc^2]/[5/3+3K/8Cc-K^3/8Cc^3], Cc >K |  |  |  |  |  |  |  |
| Cc =[2Pi^2E/Fy]^0.5 |  | K =12Lw/rx | Ca | C = P/A | Ba=Fy/Sf | B=12M/Zx |  |  |  |  |
| 126.10 |  | 13.60 | 20,968 | 8,275 | 24,000 | 13,583 |  |  |  |  |
| OK |  |  |  | 1>C/Ca+B/Ba = | 0.96 | OK |  |  |  |  |
| WALE SPLICE |  | V = W[Lj-Di/6\/2 = |  | 125,845 | lbs |  |  |  |  |  |
| A325 bolts, single shear, "N" STD (threads in shear plane) |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Plate min |  |  |  |  |
| Bolt Size | Bolt | Fv = Bolt | Pb = Bolt | Min No. | Use No. | Thickness |  |  |  |  |
| Diameter | Area | Allowable | Allowable | of Bolts | of bolts | inch |  |  |  |  |
| inch | sqin | Stress, psi | Load, lbs | each | each | 0.418 |  |  |  |  |
| 1 1/8 | 0.994 | 21,000 | 20,874 | 6.0 | 6 | 1/2 |  |  |  |  |
| BUTT WELD WALE WEB |  |  |  |  |  |  |  |  |  |  |
| Steel | Size of | No. of | Weld | Lw=Weld | Weld | Weld |  |  |  |  |
| Grade | butt | Welds | Stress | length, Lw | Allowable | Shear |  |  |  |  |
| Fy, psi | Weld, in | each | 0.6Fy, psi | Dj-2, inch | Load, lbs | lbs |  |  |  |  |
| 36,000 | 1/4 | 1 | 21,600 | 28.01 | 151,254 | 125,845 |  |  |  |  |
|  |  |  |  |  |  | OK |  |  |  |  |
| WEB BEARING |  |  |  |  |  |  |  |  |  |  |
|  | Bearing = largest Q = |  |  | 282,998 | lbs |  |  |  |  |  |
|  | Bearing stress = Q/(web length x web thickness); 0.75Fy psi maximum |  |  |  |  |  |  |  |  |  |
|  | W30x116 | Dj, inch | tw, inch | Stress |  |  |  |  |  |  |
|  |  | 30.01 | 0.565 | 16,690 | psi | OK |  |  |  |  |
| LOWER | shield | Beam |  | M, ft-lb |  |  |  |  |  |  |
| WALE | length | Span |  | QbLi^2/8 | P |  |  |  |  |  |
| LENGTH | Ls | Li=Ls-Dj | Qb | -VDj/24 | QtLj/2 | Es |  |  |  |  |
|  | ft | ft | lb/ft | ft-lb | lb | psi |  |  |  |  |
|  | 30.00 | 26.94 | 18,507 | 1,510,489 | 166,560 | 2.90E+07 |  |  |  |  |
| Beam size | Zx, cin | A, sqin | Depth, Di in | r(x), in | Web Thick | Stl Grade | Sf: |  |  |  |
| W36x300 | 1260 | 88.3 | 36.74 | 15.2 | 0.945 | 36,000 | 1.5 |  |  |  |
| Cc =[2Pi^2E/Fy]^0.5 |  | KL/r | Ca | C | Ba | B |  |  |  |  |
| 126.10 |  | 23.68 | 20,368 | 1,886 | 24,000 | 14,386 |  |  |  |  |
| OK |  |  | 1>C/Ca+B/Ba |  | 0.69 | OK |  |  |  |  |
|  |  |  |  | Mmax |  |  |  |  |  |  |
| LOWER | Span | Wale Length | Qb | QbLw^2/8 | V | P | Es |  |  |  |
| WALE | Width, Lj | Lw=Lj-Di/6 |  | -PDj/36 | QbLw/2 | QbLs/2 | Modulus |  |  |  |
| WIDTH | ft | ft | lb | ft-lb | lb | lb | psi |  |  |  |
|  | 18.00 | 11.88 | 18,507 | 43,001 | 109,899 | 277,600 | 2.90E+07 |  |  |  |
| Beam size | Zx, cu in | A, sqin | Depth, Dj in | r(x), in | Web Thick | Stl Grade | Sf: |  |  |  |
| W36x300 | 1260 | 88.3 | 36.74 | 15.2 | 0.945 | 36,000 | 1.5 |  |  |  |
| Cc =[2Pi^2E/Fy]^0.5 |  | KLw/r | Ca | C | Ba | B |  |  |  |  |
| 126.10 |  | 9.38 | 21,186 | 3,144 | 24,000 | 410 |  |  |  |  |
| OK |  |  | 1>C/Ca+B/Ba |  | 0.17 | OK |  |  |  |  |
| WALE SPLICE |  | V = |  | 109,899 | lbs |  |  |  |  |  |
| A325 bolts, single shear, "N" STD (threads in shear plane) |  |  |  |  |  |  |  |  |  |  |
|  | Bolt Size | Bolt | Fv = Bolt | Pb = Bolt | Minimum No. |  |  |  |  |  |
|  | Diameter | Area | Allowable | Allowable | of Bolts | Use |  |  |  |  |
|  | inch | sqin | Stress, psi | Load, lbs | each | each |  |  |  |  |
|  | 1 1/4 | 1.227 | 21,000 | 25,771 | 4.26 | 8 |  |  |  |  |
| Shear plate |  |  |  |  |  |  |  |  |  |  |
|  | Weld plate to beam w/ fillet all around |  |  |  |  |  |  |  |  |  |
| Steel | Safety | Size of | No. of | Weld | Lw=Weld | Weld | Plate min |  |  |  |
| Grade | factor | Butt | Welds | Stress | length, Lw | Load | Thickness |  |  |  |
| Fy, psi | Sf | Weld, in | each | Fy, psi | Dj-2, inch | lbs | inch |  |  |  |
| 36,000 | 1.65 | 3/8 | 1 | 18,000 | 34.74 | 234,495 | 0.574 |  |  |  |
|  |  |  |  |  |  | OK | 5/8 |  |  |  |
| WEB BEARING |  |  |  |  |  |  |  |  |  |  |
|  | Bearing = largest Q = |  |  | 277,600 | lbs |  |  |  |  |  |
|  | Bearing stress = Q/(web length x web thickness); 0.75Fy psi maximum |  |  |  |  |  |  |  |  |  |
|  | W36x300 | Dj, inch | tw, inch | Stress |  |  |  |  |  |  |
|  |  | 36.74 | 0.945 | 7,996 | psi | OK |  |  |  |  |