| EXAMPLE |  |  | BRACED SHEET PILES |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Active | Passive | Sheet pile |  |  |  |
| H, trench | St, wale | Gs, soil | K, active | F, angle | Ap, angle | Fy, steel | Sf, saftey | Ge, live | Fa, steel |
| depth | depth | density | constant | friction | of friction | yield | factor | surcharge | allowable |
| ft | ft | pcf | ? : 1 | deg | deg | psi | ? : 1 | psf | psi |
| 31.00 | 10.00 | 100 | 0.8 | 30.00 | 30 | 50,000 | 1.50 | 72 | 33,333 |
| Mx,pile | Mt, pile | Sxmin, pile | PILE |  | L, pile min |  | Goalseek Msum=0 by changing Y |  |  |
| Moment | Moment | section | PZ 22 |  | length |  |  | Msum=0 | Y |
| ft-lb | Cant, ft-lb | cin | Sx, cin |  | ft |  |  | ft-lb | ft |
| 29,789 | 19,352 | 10.72 | 18.1 | OK | 36.79 |  |  | 0.00 | 5.79 |
| Ca | Cp | Ga | Gp | Ws | Wt | Qt | R | X |  |
| ? : 1 | ? : 1 | lb/cft | lb/cft | psf | lb/ft | lb/ft | lb/ft | ft |  |
| 0.33 | 3.00 | 33.33 | 300 | 827 | 22,733 | 17,704 | 5,029 | 16.83 |  |
| 0.2H | St | Ws = KGaH |  |  |  | H = Hbox-Sb+1 |  |  | Gp = CpGs |
|  | Qt | Wt = (0.8Ws +Ge)H |  |  |  | Sxmin =12 Mmax/Fa |  |  | L = H+Y |
| H |  | R = GpY^2/2 |  |  |  | Ca = [1-sinAf]/[1+sinAf] |  |  |  |
| Wt |  | P = W - R |  |  |  | Cp = [1+sinAf]/[1-sinAf] |  |  |  |
|  |  | Msum = 0 = Wt(H/2-St)-R(H+2Y/3-St) |  |  |  | Ga = CaGs |  |  |  |
| 0.2H |  | X = [Qt-WsH/10-0.2HGe]/[Ws+Ge] |  |  |  |  |  |  |  |
|  |  | Mt = 0.75[WsH(St-H/7.5)/10+Ws(St-0.2H)^2/2]+GeSt^2/2 |  |  |  |  |  |  |  |
| Y | R | Mx = 0.75[Qt(X+0.2H-St)-WsH(X+H/15)/10-WsX^2/2]-Ge(X+0.2H)^2/2 |  |  |  |  |  |  |  |
| Fa = Allowable steel bending stress |  |  |  |  | Qt = Wale reaction per lt |  |  |  |  |
| Ga = Active soil pressure |  |  |  |  | R = Total soil passive pressure |  |  |  |  |
| Gp = Passive siol pressure |  |  |  |  | St = Ground to wale heigth |  |  |  |  |
| H = Trench depth |  |  |  |  | Sx = Minimum sheet pile section modulus |  |  |  |  |
| L= Minimum sheet pile length |  |  |  |  | Wt = Total soil pressure per lft of wall |  |  |  |  |
| Mx = Bending moment where shear is zero |  |  |  |  | X = heigth from ground @ Mx, shear = 0 |  |  |  |  |
| Msum = 0 = Conservation of moments |  |  |  |  | Y = Sheet pile toe embedment |  |  |  |  |
| Mt = Cantilever moment about the wale |  |  |  |  |  |  |  |  |  |
| DESIGN COFFERDAM WALES |  |  |  |  |  |  |  | Length |  |
| WALE LENGTH |  |  |  |  |  |  |  |  |  |
| Side | Span | Qi, lb | Mmax = P[Li(Li-Dj/12)/4-Li^2/8-LwDi/48] |  |  |  |  |  |  |
| Length,Li | Ls=Li-Dj/6 | PLj/2 | Mmax | P | Es | Width |  |  |  |
| ft | ft | lb | ft-lb | lb/lf | psi |  |  |  |  |
| 30.58 | 28.08 | 173,351 | 1,551,303 | 17,704 | 2.90E+07 |  |  |  |  |
| Beam size | Zx, cu in | A, sqin | Depth, Di in | r(x), in | Web Thick | Stl Grade | Sf: |  |  |
| W36x230 | 943 | 67.6 | 35.9 | 14.9 | 0.76 | 36,000 | 1.5 |  |  |
| Cc =[2Pi^2E/Fy]^0.5 |  | KL/r | Ca | C | Ba | B |  |  |  |
| 126.10 |  | 24.63 | 20,307 | 2,564 | 24,000 | 19,741 |  |  |  |
| OK |  |  | 1>C/Ca+B/Ba |  | 0.95 | OK |  |  |  |
| WALE WIDTH |  |  |  |  |  |  |  |  |  |
| Side | Span | Qj | Mmax |  |  |  |  |  |  |
| Width, Lj | Lw=Lj-Di/6 | PLi/2 | PLw^2/8 | Es | Wale Length |  |  |  |  |
| ft | ft | lb | ft-lb | psi | Lw=Lj-Di/6 |  |  |  |  |
| 19.58 | 13.60 | 270,722 | 409,314 | 2.90E+07 | 13.60 |  |  |  |  |
| Beam size | Zx, cu in | A, sqin | Depth, Dj in | r(x), in | Web Thick | Stl Grade | Sf: |  |  |
| W30x116 | 378 | 34.2 | 30.01 | 12 | 0.565 | 36,000 | 1.5 |  |  |
| Cc =[2Pi^2E/Fy]^0.5 |  | KLw/r | Ca | C | Ba | B |  |  |  |
| 126.10 |  | 13.60 | 20,968 | 7,916 | 24,000 | 12,994 |  |  |  |
| OK |  |  | 1>C/Ca+B/Ba |  | 0.92 | OK |  |  |  |
|  |  | V = PLw/2 = |  | 120,387 | lbs |  |  |  |  |
| A325 bolts, single shear, "N" STD (threads in shear plane) |  |  |  |  |  |  |  |  |  |
|  | Bolt Size | Bolt | Fv = Bolt | Pb = Bolt | Minimum No. |  |  |  |  |
|  | Diameter | Area | Allowable | Allowable | of Bolts | Use |  |  |  |
|  | inch | sqin | Stress, psi | Load, lbs | each | each |  |  |  |
|  | 1 1/8 | 0.994 | 21,000 | 20,874 | 5.77 | 8 |  |  |  |
| Shear plate |  |  |  |  |  |  |  |  |  |
|  | Weld plate to beam w/ fillet all around |  |  |  |  |  |  |  |  |
| Steel | Size of | No. of | Weld | Lw=Weld | Weld | Plate min |  |  |  |
| Grade | Butt | Welds | Stress | length, Lw | Load | Thickness |  |  |  |
| Fy, psi | Weld, in | each | 0.6Fy, psi | Dj-2, inch | lbs | inch |  |  |  |
| 36,000 | 1/2 | 1 | 10,800 | 28.01 | 151,254 | 0.337 |  |  |  |
|  | OK |  |  |  | OK | 1/2 |  |  |  |
| WEB BEARING |  |  |  |  |  |  |  |  |  |
| Bearing = largest Q = |  |  | 270,722 | lbs |  |  |  |  |  |
| Bearing stress = Q/(web length x web thickness); 0.75Fy psi maximum |  |  |  |  |  |  |  |  |  |
| Waler | Dj, inch | tw, inch | Stress |  |  |  |  |  |  |
| W30x116 | 30.01 | 0.565 | 15,966 | psi | OK |  |  |  |  |