| EXAMPLE |  |  | CANTILEVER SHEET PILES |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Gs = Soil | F = friction | H = Bank | Ge = live | Fy =steel | Sf =saftey | Es=Steel | Fa | F |  |
| density | angle | cut heigth | surcharge | yield | factor | Modulus | allowable | angle in |  |
| lb/cf | degrees | ft | lb/sf | psi | ? : 1 | psi | psi | radians |  |
| 100 | 30 | 11.00 | 72 | 50,000 | 1.50 | 2.90E+07 | 33,333 | 0.52 |  |
|  | Sheet | PILE: | PILE: | PILE: | PILE: | Goalseek V = 0 by changing X: |  |  |  |
| Mmax | Sxmin | PZ22 | Ix | Length, Lp | Deflection |  | V= 0 = | X, ft |  |
| ft-lb | cin | Sx cin/ft | qin/ft | ft | Dm, in |  | lbs | ft |  |
| 27,068 | 9.74 | 18.10 | 84.4 | 23.95 | 2.07 |  | 0.00 | 6.52 |  |
|  |  | OK |  |  |  |  |  |  |  |
| Ca=active | Cp=passive |  | Ga | Gp |  | Goalseek Msum= 0 by changing Y: |  |  |  |
| coeficient | coeficient |  | lb/cft | lb/cft |  |  | Msum = 0 | Y, ft |  |
| 0.33 | 3.00 |  | 33 | 300 |  |  | 0.00 | 12.95 |  |
| N | P | R | Q | N+P+Q-R |  | J | Z | Ld |  |
| lb | lb | lb | lb | = 0, check |  | ft | ft | inches |  |
| 1,724 | 9,558 | 25,143 | 13,861 | 0.00 |  | 17.52 | 2.50 | 187 |  |
|  |  |  |  | H |  |  |  |  |  |
|  |  | N |  |  | Lp |  |  |  |  |
|  | P |  |  |  |  | Dm = deflection at the top of pile, inches |  |  |  |
|  |  |  |  | Y |  | F = Soil friction angle |  |  |  |
|  |  |  | R |  |  | Fa = Allowable bending stress, Fy/S.F. |  |  |  |
|  | Q |  | Z |  |  | Fy = Steel yield strength |  |  |  |
|  |  |  |  |  |  | Ga = Active soil pressure, equivalent fluid |  |  |  |
|  | Ca = [1-SinF]/[1+SinF] |  |  |  |  | Gp = Passive soil pressure |  |  |  |
|  | Cp = [1+SinF]/[1-SinF] |  |  |  |  | Gs = Soil Unit Weight |  |  |  |
|  | Dm = PL^3/(15EI)+(Ws/12)L^4/(8EI) |  |  |  |  | H = Bank supported height |  |  |  |
|  | Ga = CaGs |  |  |  |  | Ix = Sheet Moment of inertia per foot |  |  |  |
|  | Gp = CpGs |  |  |  |  | J = Height top V=0 from top of pile |  |  |  |
|  | H = St + 1 |  |  |  |  | Ld = Height used for deflection calc |  |  |  |
|  | J = H+X |  |  |  |  | Lp = Minimum pile length |  |  |  |
|  | Ld = 12(H+0.7X) = |  |  |  |  | Mmax = Maximum bending moment |  |  |  |
|  | Lp = H+Y |  |  |  |  | Msum = 0 = Conservation of moments |  |  |  |
|  | Mmax @ v=0 = GeJ^2/2+GaJ^3/6-GpX^3/6 |  |  |  |  | N = Surcharge sheet loading |  |  |  |
|  | Msum = 0 = NLp/2+PLp/3+QZ/3-RY/3 |  |  |  |  | P = Total active soil force |  |  |  |
|  | N = GeLp |  |  |  |  | Q = Tip passive reaction force |  |  |  |
|  | P = GaLp^2/2 |  |  |  |  | R = Soil passive pressure reaction force |  |  |  |
|  | Q = GpZ[H+2Y]/2 |  |  |  |  | Sf = Saftey factor |  |  |  |
|  | R = GpD^2/2 |  |  |  |  | Sx = Minimum sheet pile section modulus |  |  |  |
|  | Sxmin = 12Mmax/Fa |  |  |  |  | X = Depth below excavation @ V = 0 |  |  |  |
|  | V = 0 = GeJ+GaJ^2/2-GpX^2/2 |  |  |  |  | Y = Pile penetration below excavation |  |  |  |
|  | Z = 2[N+P-R]/[Gp(H+2Y)] |  |  |  |  | Z = Projected Q heigth |  |  |  |